Documentation from :F5:BG59B

Intel insite Reference: BG59

File Name: BG59B.DOC

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;

Series-III i8087 Linkage Module, Large Compilation Model                    ;

;

Written By:                                                                 ;

Mike Silverstone                                                            ;

Brunswick Defense                                                           ;

3333 Harbor Blvd.                                                           ;

Costa Mesa, California  92626-1009                                          ;

Revised: 27 Apr 82                                                          ;

;

This module links the interrupt of the i8087 Numeric Data Processor on an   ;

iSBC-337 Multimodule Math Board installed on the Series-III RPB-86 board to ;

the FORTRAN-86 and PASCAL-86 math exception handlers and the RUN program    ;

default math exception handler.  ISIS-II RUN, V1.0 and V1.3, do not         ;

recognize the existance of an i8087 Numeric Data Processor in the           ;

Series-III Microcomputer Development System.                                ;

;

It is assumed that an Intel iSBC-337 Multimodule Math Board has been        ;

installed on the Series-III RPB-86 Board in accordance with the 142887-001  ;

iSBC-337 Multimodule Numeric Data Processor Hardware Reference Manual.  It  ;

is also assumed that a wire has been connected between P2-1, the math       ;

interrupt output on the iSBC-337 Multimodule Math Board, and E74, the       ;

programmable interrupt controller interrupt 7 input on the RPB-86 Board.    ;

;

Three separate parts are in this module:                                    ;

1. The first part loads the math interrupt procedure entry point into     ;

the RPB-86 Board interrupt vector array.                               ;

2. The second part contains the math interrupt procedure.  It generates   ;

a software interrupt 16 which is the standard Intel Numeric Data       ;

Processor exception handler entry point.                               ;

3. The third part modifies the ISIS-II RUN program default PIC mask to    ;

acknowledge the presence of the math interrupt on the RPB-86 Board     ;

PIC interrupt 7 input.                                                 ;

;

This procedure must be called at the beginning of the user program to link  ;

the i8087 to the Series-III Microcomputer Development System before any     ;

floating point operations occur.  The linkage of the LINK87 procedure to    ;

the user program will set up the proper PIC mask, and link the math         ;

interrupt vector and the math interrupt procedure to the user program.      ;

;

NOTE:                                                                       ;

This module is compatible with the PL/M-86 Medium and Large models of       ;

compilation.  With suitable modification it will also be compatible with    ;

the PL/M-86 Small and Compact models of compilation.                        ;

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;