FIXOBJ - A Load-address Translation Program

for iAPX 86,88 Absolute Object Files

John Hall, Eastman Kodak Co., Rochester, New York

When a program is loaded into the dual-port memory of a computer such

as an iSBC-86/12A, it can be necessary to load a program at a

different address than that at which it will be executed.  This can

occur because, in a multi-processor system, the Multibus address of a

block of dual-port on-board memory can be made different from its

local "on-board" address to prevent conflicts with the memory of

other processors in the same system.

FIXOBJ is a utility program that will process an iAPX 86,88 absolute

object file, changing all load addresses by a specified amount.  It

runs on Intellec Series II and Series III computers, under the

ISIS-II operating system.

The actual code is undisturbed.  It still must be run at the address

that it was located at by LOC86.  Only the load address fields of the

PEDATA and PIDATA object records are changed.

The user is strongly urged to study Appendix A of the iAPX 86,88

Family Utilities User's Guide (Intel Order # 121616-002).  This

appendix, "iAPX 86,88 Absolute Object File Formats" describes the

format of these files in detail.

Invocation:

[directory]FIXOBJ[switches] [input file] TO [output file]

[switches] are:

/O=[+|-]nnnnn[H]  - 	Specifies the offset to be added

to or subtracted from the load

addresses.  This is evaluated as a

signed 	20-bit number.  The leading

sign and trailing "H" are optional.

The number is made up of from one to

five hexadecimal digits.  Default

value is 00000H.

/S=nnnnn[H]	  -	Translates only those records whose

load addresses are greater than or

equal to "nnnnn".  This is a 1-5 byte

hexadecimal number that is evaluated

as a 20-bit unsigned number.  The

trailing "H" is optional. Default

value is 00000H.

/E=nnnnn[H]	  -	Translates only those records whose

load addresses are less than or

equal to "nnnnn".  This is a 1-5 byte

hexadecimal number that is evaluated

as a 20-bit unsigned number.  The

trailing "H" is optional.  Default

value is FFFFFH.

FIXOBJ - Translation Program for Object Files              Page 2

EXAMPLE:

-FIXOBJ/O=40000/E=FFFFH :F1:FOOBAR.LOC TO :F1:FOOBAR.LOD

All load records in the absolute object module FOOBAR.LOC

that have load addresses between 00000H and 0FFFFH will

be given corresponding load addresses between 40000H and

4FFFFH in the output file, FOOBAR.LOD.

WARNING:

FIXOBJ only looks at the load address of the record when it

determines whether to apply the offset to that record.  It is

possible for a long record to extend over the address given

by the /S= or /E= switch.  In the example above, a record

with the load address of FFFEH that is 200H bytes long (and

extends beyond FFFFH) will have its load address translated,

even though part of the record is outside of the selected

range.

FIXOBJ UTILITY DISK

John Hall,  Eastman Kodak Co,  Rochester, NY

- FILES -

FIXOBJ		- executable FIXOBJ utility

FIXOBJ.DOC	- documentation for FIXOBJ

FIXOBJ.CSD	- SUBMIT file to compile, link and locate

FIXOBJ

FIXOBJ.PLM	- PL/M-80 source for FIXOBJ

TSTFIX.CSD	- SUBMIT file to test the utility.

TSTFIX.DAT	- Data file for the test.

TSTFIX.SAM	- Sample output file from test run

- TEST PROCEDURE -

1.	Make sure FIXOBJ, TSTFIX.CSD, and TSTFIX.DAT are

all on device :F1:

2.	SUBMIT :F1:TSTFIX

3. Compare the new file, TSTFIX.OUT with TSTFIX.SAM.  If they

match, then FIXOBJ is operating correctly.