FILE: BG78.DOC

REVISED: 28 SEP 1982

BY: WILLIAM J. HINKLE, COMTEC INC.

THE SQ PROGRAM IS A UTILITY TO BE USED ON THE INTEL MDS800 AND INTELLEC

DEVELOPMENT SYSTEMS.  THE UTILITY OPERATES AS A "SUBMIT QUICK" COMMAND IN

THAT THE SYSTEM IS CONTROLLED BY A COMMAND STRING, AS IN SUBMIT, BUT WITHOUT

THE NECESSITY OF EDITING (AND LATER DELETING) A CSD FILE.  THE SQ UTILITY

IS THEREFORE MOST USEFUL FOR IMPLEMENTING SHORT, ONE-TIME COMMAND STRINGS

RATHER THAN PERMANENT COMMAND STRINGS (AS FOR LINK/LOCATE, ETC.).  THE SQ

UTILITY PERMITS THE OPERATOR TO ENTER, FOLLOWING THE SQ COMMAND INVOCATION,

A STRING OF ISIS COMMANDS SEPARATED BY SEMICOLONS.  THE LENGTH OF EACH

INDIVIDUAL COMMAND CANNOT, AS USUAL, EXCEED THE LENGTH OF AN ISIS LINE BUFFER

(122 CHARACTERS).  NOTE THAT THE LENGTH OF THE FIRST COMMAND INCLUDES THE

TEXT OF THE SQ INVOCATION COMMAND, UNLESS THIS IS FOLLOWED BY A SEMICOLON.

THE TOTAL LENGTH OF THE COMMAND LINE (INVOCATION PLUS ALL COMMANDS) CAN BE

ANY LENGTH -- TEXT WILL BE PROCESSED 122 CHARACTERS AT A TIME, SO THAT THERE

WILL BE A DELAY FOR DISK ACCESS IN BETWEEN 122 CHARACTER SEGMENTS OF A LONG

COMMAND LINE.  TERMINATION OF THE COMMAND LINE WITH <RETURN> OR <ESCAPE>

CAUSES SQ TO STORE THE COMMANDS IN THE STRING AS SEPARATE ISIS COMMANDS IN

A SQ.CS FILE (MUCH LIKE SUBMIT WITHOUT PARAMETER SUBSTITUTION), TERMINATE

THIS FILE WITH AN SQ RESTORE INVOCATION, AND PASS CONSOLE COMMAND TO THE

SQ.CS FILE.  THE COMMANDS THEN EXECUTE SEQUENTIALLY, AND THE SQ RESTORE

COMMAND DELETES THE CS FILE AND RESTORES CONSOLE COMMAND TO WHATEVER DEVICE

WAS ASSIGNED AS THE :CI: (CONSOLE INPUT) WHEN SQ WAS INVOKED. THE SQ.CS FILE

IS, BY DEFAULT, CREATED ON :F1: BUT ANOTHER WORKFILE DEVICE CAN BE SPECIFIED

IN A MANNER SIMILAR TO DIR:  A DECIMAL NUMBER DIRECTLY FOLLOWING "SQ" WILL BE

INTERPRETED AS AN ALTERNATE DRIVE NUMBER FOR THE SQ.CS WORKFILE.

EDITING CONSIDERATIONS:

-- ALL ISIS LINE EDITING COMMANDS MAY BE USED NORMALLY (CONTROL-X, ETC.)

-- THE CONTROL-E FUNCTION MAY BE USED AS IN A SUBMIT FILE, BUT IN ORDER TO

PLACE IT IN THE COMMAND STRING THE CONTROL-E CHARACTER MUST BE LITERALIZED;

THAT IS, THE CONTROL-E KEY MUST BE PRECEDED BY THE CONTROL-P KEY.

-- SEMICOLONS MAY BE USED IN A COMMAND BY USING TWO SEQUENTIAL SEMICOLONS ;;

-- CARRIAGE RETURNS AND LINE FEEDS CANNOT BE LITERALIZED

SOME EXAMPLES:

-SQ DELETE :F1:SQ.BAK;PLM80 :F1:SQ.PL8 PRINT(:LP:);SUBMIT :F1:SQLINK<RETURN>

(THE DELETE AND PLM80 UTILITIES WILL BE INVOKED, FOLLOWED BY A SUBMIT FILE.)

-SQ 4 DIR ; DIR 1; DIR 4 <RETURN>

(THE DIR UTILITY WILL BE INVOKED THREE TIMES FOR DIFFERENT DEVICES. NOTE THAT

THE SQ CS WORKFILE IS PLACED ON :F4: (DRIVE 4).)